Notes on Address Classes

- Thoroughfare Classes

  - Specifies a location by reference to a road or other access route by which the address feature can be reached

  - This can be a road, railroad or river

  - In most addresses the thoroughfare is designated by a Complete Street Name and a Complete Address Number

  - Consists of four classes

    - Numbered Thoroughfare class

    - Intersection Address

    - Two Number Address Range

    - Four Number Address Range

- Numbered Thoroughfare class

  - Has a Complete Address Number and Complete Street Name

  - Includes most business and residential addresses

  - Sometimes preceded by Landmark Names

  - Sometimes preceded by Complete Place Names

    - Neighborhood Names

    - Housing project Name

    - Puerto Rican urbanization

    - The place name type for these instances must be “community”

- Intersection Address

  - Must include two or more Complete Street Names, each separated by a Separator element

  - Useful for recording events occurring in the street

    - Accidents

    - Infrastructure locations

  - May be preceded by a Complete Landmark Name or Complete Place Name

  - Complete Street Names may be rewritten in any order

  - An intersection corner address can only include two Complete Street Names

- Two Number Address Range

  - Must include two Complete Address Numbers separated by a hyphen

  - Two Complete Address Numbers must be followed by a Complete Street Name

  - Includes a set of two Complete Address Numbers

    - Represent the low and high values of a continuous set of Complete Address Numbers

    - The first Complete Address Number represents the low end of the range

    - The second Complete Address Number represents the high end of the range

    - The two numbers are separated by a hyphen

  - A range that refers to Complete Address Numbers on one side of the thoroughfare will have the same parity: odd or even

  - A range can begin or end with a Complete Address Number that has a suffix or prefix

  - The Address Range Type can be used to show whether a Two Number Address Range represents an actual or potential range

  - The Address Range Parity attribute can be used to show whether a Two Number Address Range includes Complete Address Numbers that are odd, even or both

  - The Address Range Side Attribute is used to if the Complete Address Numbers are on the right side, left side or both sides of the segment

  - The Address Range Directionality Attribute may be used to show if the Complete Address Numbers in the range increase with or against the directionality of the segment

  - The Address Range Span attribute may be used to show whether the range spans part of one segment, an entire segment, multiple segments, or the entire length of a thoroughfare

  - A specific address range can be related to  a specific  in a specific transportation model through the use of these attributes:

    - Transportation System Name

    - Address Transportation System Authority

    - Address Transportation Feature

    - Related Transportation Feature ID

  - Ranges should not be confused with hyphenated address numbers that denote a single site

  - May be preceded by a Complete Landmark Name or Complete Place Name

- Four Number Address Range

  - Must include four Complete Address Numbers followed by a Complete Street Name

  - Follows the structure established by the Census Bureau for TIGER/Line street segment address ranges

  - Important for municipal operations , emergency dispatch and geocoding

  - Includes four Complete Address Numbers representing the low and high end of the Complete Address Range

  - Left and Right ranges will have different parities